Caution for Installation

# MTV-53S80H COLOR BOARD CAMERA INSTRUCTION

1. Avoid places where there is direct sunlight. When using the camera outside.

2. Be careful, never let any water in this equipment.

3. Do not directly touch the CCD element.

If necessary, use a soft cloth moistened with alcohol to wipe off the dust.

4. When the camera installs in dust or wet environment, please puts into the camera housing.

5. Be extra careful not to shake the camera.

6. Avoid places where temperatures exceed 50 ℃ or more.

7. Be extra careful not to mistake plug in connect.

8. The camera are only operated with regulative adaptor DC 12V, AC to DC non-regulative adaptor may cause camera damaged.

#### SPECIFICATION

| MODE NO. |  |  | MTV-53S80HN | MTV-53S80HP |
| --- | --- | --- | --- | --- |
| TV SYSTEM |  |  | NTSC | PAL |
| IMAGE SENSOR |  |  | 1/3-inch I.L. CCD Image Sensor |  |
| CCD TOTAL PIXELS |  |  | ~ 410K | ~ 470K |
| SYNC SYSTEM |  |  | Internal / Line lock(3.3V VD pulse input) |  |
| MINIMUM ILLUMINATION |  |  | 0.012 Lux F1.2 / 20 IRE AGC on 0.0014Lux F1.2 / 20 IRE SENS-UP (32X) |  |
| LENS & VIEW ANGLE |  |  | 3.6mm F2.0 / 92 |  |
| RESOLUTION |  |  | 540 TVL |  |
| WHITE BALANCE |  | Mode | ATW / AWC |  |
|  |  | Range | 3200 ~ 10000 ºK |  |
| S / N RATIO TYPICAL ( MAX ) |  |  | 50dB (MIN) / 58dB (TYP) (Gamma, Aperture, AGC OFF) |  |
| AUDIO OUTPUT |  |  | Build in 60 dB Gain audio amplifier with Low impedance output |  |
| FRAME INTEGRRATION |  |  | ON / OFF (Preset 4X ,Max 32X by program set) |  |
| H - MIRROR FUNCTION |  |  | OFF / ON |  |
| V - MIRROR FUNCTION |  |  | OFF / ON |  |
| B.L.C. FUNCTION |  |  | ON / OFF |  |
| ZOOM FUNCTION |  |  | OFF / ON |  |
| ENHANCE |  |  | HIGH / MID |  |
| FLICKLESS |  |  | OFF / ON |  |
| COLOR BAR |  |  | OFF / ON |  |
| IR-Cut-FILTER SW |  |  | ON / OFF |  |
| AUTO IRIS |  |  | A.E.S. / DC-IRIS |  |
| ELECTRONIC SHUTTER |  | A.E.S. | 1/60~1/120,000 sec. | 1/50~1/120,000 sec. |
|  |  | FIX. | 1/60(50),1/100,1/250,1/500,1/1000,1/2000,1/4000,1/10000sec. |  |
| SDNR |  |  | Built-in SDNR (Super Digital Noise Reduction) |  |
| VIDEO OUTPUT | ANALOGY |  | 1.0V p-p composite video at 75 ohm |  |
|  | DIGITAL |  | ITU-601 |  |
| GAMMA CORRECTION |  |  | 0.45 / 1.0 |  |
| GAIN CONTROL |  |  | AGC ON / OFF |  |
| OPERATION TEMPERATURE |  |  | -20℃ TO 50℃ |  |
| OPERATIONAL HUMIDITY |  |  | within 85﹪RH |  |
| POWER SUPPLY |  |  | DC12V±1V / 150mA |  |

＊The specifications and appearance of the product may changed without notice.

Dimension: Unit (mm / ±0.3mm)

| JP1 |  | JP2 | ITU-656 (MID) | ITU-601 (HIGH) |
| --- | --- | --- | --- | --- |
| 1 | +12V | 1 | DCK | DCK |
| 2 | GND | 2 | D0 | D0 |
| 3 | VIDEO | 3 | D1 | D1 |
| 4 | MONITOR | 4 | D2 | D2 |
| 5 | GND | 5 | D3 | D3 |
|  |  | 6 | D4 | D4 |
| JP3 |  | 7 | D5 | D5 |
| 1 | S0 | 8 | D6 | D6 |
| 2 | S1 | 9 | D7 | D7 |
| 3 | S2 | 10 | NC | HD |
| 4 | GND | 11 | NC | VD |
|  |  | 12 | GND | GND |

| JP4 |  | JP5 |  | JP8 |  | JP10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | AGC | 1 | VD in | 1 | IR-SW_A | 1 | +3.3V |
| 2 | ATW/AWB | 2 | GND | 2 | IR-SW_B | 2 | RX |
| 3 | H-MIRROR |  |  |  |  | 3 | TX |
| 4 | V-MIRROR | JP6 |  | JP9 |  | 4 | GND |
| 5 | BLC | 1 | MIC | 1 | Audio out | FOR MANUFACTURE NOT RELEASE |  |
| 6 | ZOOM | 2 | GND | 2 | GND |  |  |
| 7 | ENHANCE |  |  |  |  |  |  |
| 8 | GAMMA | JP7 |  | JP11 |  |  |  |
| 9 | COLOR BAR | 1 | DMP- | 1 | IR-SW ON/OFF |  |  |
| 10 | AES/ALC | 2 | DMP+ | 2 | F/I ON/OFF |  |  |
| 11 | FLC | 3 | DRV+ | 3 | GND |  |  |
| 12 | GND | 4 | DRV- |  |  |  |  |

＊ At the low light conditions change to monochrome image, JP11_PIN-1 must be connect to GND